## Executive Summary :

The organization brought me in to perform an independent penetration test which focused on evaluating the security position of their entire IT infrastructure. The research aimed to identify security vulnerabilities that attackers could use while providing actionable guidance to minimize business exposure. The testing focused on a specific set of endpoints which belonged to the `192.168.10.0/24` subnet and included host discovery and service enumeration and vulnerability analysis and controlled proof-of-concept exploitation.

Business Impact :

1.The system faces two major security threats because of these vulnerabilities.

2.The exploitation of MS17-010 vulnerability leads to unauthorized data access and financial loss and regulatory penalties and damage to reputation. A compromised workstation provides attackers with access to launch network-wide attacks.

3.Service Availability:  The CentOS DoS vulnerability endangers service uptime. Extended outages disrupt operations which violate SLAs while damaging customer trust and resulting in non-compliance issues.

Risk Level and Urgency:

MS17-010 – Critical: Immediate action is required. The vulnerability enabled WannaCry ransomware to spread globally and it continues to pose a major threat to unsecured systems that do not receive security updates.

RegSvc DoS – High: While less severe, it still demands timely remediation due to the risk of operational disruption.

## Detailed Technical Report:

Scope:

The penetration test scanned the entire IPv4 subnet `192.168.10.0/24` which contained all operational hosts and network equipment and accessible services. The activities included host discovery and TCP/UDP service enumeration and vulnerability scanning and authenticated and unauthenticated testing and controlled non-destructive proof-of-concept exploitation. The testing began after receiving written authorization from the participants and all critical system breakdowns needed to be reported right away. The final report includes evidence, remediation priorities, and verification recommendations.

Methodology:

Testing was performed from Kali Linux and followed a structured approach:

1. The reconnaissance phase employs Nmap ICMP/TCP sweeps and ARP discovery to identify active hosts and their reachable network ports.

2. The enumeration process consists of Nmap version detection and banner grabbing and SMB/RPC/HTTP/SSH enumeration to detect active services.

3. The process of vulnerability analysis involves matching system versions against security advisories through automated tools and manual verification to detect system weaknesses.

4. The testing process includes Hydra for verifying online passwords through throttling and John the Ripper for offline hash decryption.

5. Exploitation: Controlled Metasploit proof-of-concept exploitation followed by manual exploitation where safer.

6. The testing phase after validation required restricted lateral movement assessments to verify if attackers could identify shared resources and access them with the same credentials between network segments.

All evidence was collected which included logs and packet captures and screenshots and PoC steps. The system encountered no data extraction incidents and no destructive actions or persistent threats occurred.

FIGURE 12:

DIAGRAM FOR THE SPECTER OR WAY OF ATTACK EXECUTED

SOURCE : BY AUTHOR

## Detailed description of the test :

In this part of the report is the detailed step-by-step guide on how the vulnerability and attack was made

FIGURE 12:

.IP SCAN ON THE NETWORK 192.168.10.0/24 FOR CHECK ACTIVE HOSTS

SOURCE: BY AUTHOR

- Here we use the command “nmap -sn 192.168.10.0/24”. This command is used to find the devices that are up and working on the subnet . so from the screen shot we can see that 5 hosts are up.

Figure 13:

DOING A NETDICOVER ON THE TARFET SUBNET

SOURCE : BY AUTHOR

- Next step is we use the command “sudo netdiscover -i eth0 -r 92.168.10.0/24”. This command helps us to find the hosts that are up and there ip address and MAC address. Above we can see 2 devices are active with IP’s 192.168.10.10 and 192.168.10.20 . the rest two are restricted for subnet broadcast and etc.

FIGURE 14:

SCANNING FOR OPEN PORTS ANS OS VERSION ON THE SERVER IP

SOURCE : BY AUTHOR

- Now we use the command “sudo nmap -sV 192.168.10.10” . this commands help us to find the OS version running and the open ports on the device with that IP. In the above screen shot you can see that ports 22,30,139 are open and the device is a SERVER running centiOS.

FIGURE 15:

SCANNING FOR OPEN PORTS AND OS VERSION ON THE DESKTOP IP

SOURCE : BY AUTHOR

- We repeat the same command as step 3 for the other IP i.e 192.168.10.20. here we can see that the IP is of the device running windows 7 professional and the open ports are 135,139,445.

FIGURE 16:

FINDING VULNERABILITIES USING NMAP ON DESKTOP

SOURCE : BY AUTHOR

- As you can see in the above screenshot we have used the command “sudo nmap ---script vuln 192.168.10.20”. this commands helps us find all the vulnerabilities present on the current device that can be exploited by the attacker. As seen above the windows 7 device is vulnerable to smb-vuln-ms17-010 vulnerability.

FIGURE 17:

TURNING ON THE METASPLOIT CONSOLE AND SEARCHING THE VULNERABILITY FOUND

SOURCE : BY AUTHOR

- Next step is to turn up Metasploit console by using command “msfconsole” and searching the vulnerability ms17.

FIGURE 18:

SELECTING AND USING THE VULNEARBILITY FOUND

SOURCE : BY AUTHOR

- As you can see above after searching use find the vulnerability next step is to use the vuln to exploit the device. By command “use exploit/windows/smb/ms_010_eternalblue”.

FIGURE 19:

SETTING THE RHOST , LHOST AND PAYLOAD AND EXECUTING THE EXPLOIT

SOURCE : BY AUTHOR

- After setting the exploit we have to set remote and local host in this situation the Remote host is the victim and Local host is the attacker in this case our machine . above are the respective IP’s shown in screenshot. Next we set the payload as shown above  by command “set payload”. Payloads can be found by command “show payloads”. Last step is to run the command “exploit” to run the exploit.

FIGURE 20;

IMAGE SHOWING THE EXPLOIT WAS SUCCESSFUL

SOURCE : BY AUTHOR

- As you can see after running the exploit we were able to get into the kernel of the windows 7 device . using the M eterpreter payload.

FIGURE 21:

USING THE COOMAND HASHDUMO TO GET HASHED PASSWORDS

SOURCE: BY AUTHOR

- Now the next step is to use the command “Hashdump ” to get the hashed passwords for the users on the device to hashed passwords for the login because windows uses hash method to encrypt the passwords. As you can see in the screenshot we have got the hashes for administrator and Lora brown in this case. Next step is to copy this and save it in a txt file for further cracking .file name to be saved as hash.txt .

FIGURE 22:

USING JOHN THE RIPPER TO CRACK JOHN THE RIPPER

SOURCE : BY AUTHOR

- Final step here is to use the tool john the ripper to crack the hash passwords. Using the command shown in the screenshot and the previously saved hash.txt file we were able to crack the password. Where the password for Lora brown is lovely and she is also the administrator for the device .

FIGURE 23:

SCANNING THE VULNERABILITY ON THE SERVER IP

SOURCE : BY AUTHOR

- Next we work on the SERVER, we use the same command ’”sudo nmap –script vuln 192.168.10.10”. we were able to find that there is a vulnerability on server which is smb-regsvg-dos. Which is DDOS type vulnerability.

- The way we used to get into the server is brute forcing by cracking into the password

- It is possible because it is a Linux based server and the administrator is root for all Linux .so using this info we can crack the password .

FIGURE 24 :

USING TOOL HYDRA TO CRACK THE SERVER PASSWORD

SOURCE:BY AUTHO

- Here in the above as you can see we use tool Hydra which to crack the server password by using command as shown in screenshot , what this does it takes root as the username and uses a list of passwords present in the tool to crack the password and save it in a txt file for use to see.

FIGURE 25:

PASSWORD FOR THE SERVER WTH USERNAME ROOT

SOURCE : BY AUTHOR

- As you can see we were able to get the password that is “superman” to for root user. this can be used to do further exploits.

Summary Of Findings :

Finding 1 — MS17-010 (EternalBlue Remote Code Execution)

Severity: Critical

CVE:** 2017-0144

Description:

MS17-010 is a critical RCE vulnerability in Microsoft’s SMBv1 protocol. The vulnerability enables attackers to transmit custom SMB packets which exploit a kernel memory corruption issue to execute SYSTEM-level code remotely without needing authentication. The system maintains the ability to spread itself automatically between different network systems. During testing, a Windows 7 host was identified with open TCP port 445 and an active SMBv1 service missing the March 2017 patch. The Metasploit testing achieved payload injection success within a simulated testing environment.

Affected Systems:

Windows 7 desktop systems in the `192.168.10.0/24` subnet offering SMBv1 services.

Potential Impact:

Attackers can achieve complete administrative control while they move between systems to steal credentials and deploy ransomware and disable security controls and compromise domain-connected systems. The attack causes extensive operational disruptions and data theft and severe damage to the organization's public image. The exploit’s self-spreading nature increases the risk to all network-connected systems.

Mitigation Steps:

1. Security Updates should be applied.

All affected Windows systems need to receive the MS17-010 patch from Microsoft as an immediate installation. Verify successful deployment through patch management tools or vulnerability assessments.

2.Disable SMBv1:

All devices need to have the outdated SMBv1 protocol disabled or removed. Use the latest versions of SMBv2 or SMBv3 because they offer enhanced security features which defend against identified vulnerabilities.

3.Restrict Network Access:

The network perimeter needs to block TCP port 445 to prevent outside attackers from using it for exploitation. The system needs to limit Small and Medium Business (SMB) users to access only approved internal systems which they need to perform their duties.

4.Network Segmentation:

The organization should place unfixable systems into separate VLANs which must have limited network access. The system design prevents attackers from shifting between networks after discovering a vulnerable system.

5.Monitor and Detect Exploits:

Implement IDS/IPS signatures capable of detecting EternalBlue-related activity and monitor logs for unusual SMB traffic or privilege escalation attempts.

6.Enforce Access Controls:

Organizations must enforce the principle of least privilege by limiting administrative rights and securing credentials because it blocks attackers from performing post-exploitation activities.

7.Continuous Scanning:

Perform network scans on a regular basis to detect hosts without security patches and SMBv1 usage so you can take immediate action to protect your network.

Finding 2 — SMB RegSvc Denial-of-Service Vulnerability

Severity: High

CWE: CWE-400 – Uncontrolled Resource Consumption

Description:

A DoS vulnerability was discovered in the SMB-related Registration Service (RegSvc) on a CentOS server. The exploitation of malicious registration or SMB packets causes resource exhaustion and memory handling errors which result in service crashes and instability. The testing process involved sending deliberately corrupted packets which resulted in unstable service operations that proved the ability to create extended service disruptions. The service faces this issue because it does not perform adequate request validation and resource management.

Affected Systems:

The CentOS servers operating in the `192.168.10.0/24` network segment offer both SMB and RegSvc services.

Potential Impact:

System uptime and availability suffer from exploitation which causes disruptions to both customer applications and internal services. The system crashes repeatedly which causes data corruption and requires users to restart their work multiple times and reduces system reliability and creates problems with service-level agreements.

Mitigation Steps

1. Apply Security Updates:

The system needs to run a full update of CentOS operating system and SMB services and all related RegSvc components. The system administrator should apply vendor-provided patches or kernel updates which fix the identified DoS vulnerabilities.

2.Harden SMB and RegSvc Configuration:

Disable or remove unnecessary SMB and registration services if not required. Review service configurations to enforce strict request validation, rate limiting, and timeout settings to prevent resource exhaustion.

3.Network Access Control:

The SMB and RegSvc services should have their access restricted to trusted hosts and internal network addresses only. Use firewall rules and network segmentation to block or restrict access to these services when they originate from untrusted or external networks.

4. The system requires the implementation of traffic filtering as a method to work with rate limiting.

Deploy network security devices or host-based firewalls to filter malformed or suspicious packets. The system needs to establish connection throttling and rate-limiting rules to prevent attackers from making excessive requests for exploitation.

5.Monitoring and Logging:

System administrators need to monitor log data for any abnormal network traffic patterns and repeated service restarts. The system needs to establish alert systems which identify resource depletion indicators to initiate immediate Denial of Service attack investigation.

6.High-Availability Measures:

The implementation of redundant systems together with load balancing techniques will reduce service interruptions while continuing to operate without interruption during security attacks.

## Conclusion:

The penetration test revealed two essential security weaknesses which stemmed from the MS17-010 EternalBlue** Windows 7 workstation flaw and the Denial-of-Service vulnerability** that attacked the CentOS RegSvc service. The two options present major threats to data protection and operational stability and regulatory adherence. The organization needs to take immediate corrective action through network segmentation and patching and improved monitoring systems and enhanced governance practices to minimize its exposure to risk. A proactive security improvement plan that runs continuously will protect against future cyber threats that will emerge.